MONTHLY PM   MicroWeigh Triple Indexer

Buddy's Kitchen — Lakeville, MN   ·   Approx. 1.5–2 hr · machine down for sections 4–6

| Field | Value | Field | Value |
| --- | --- | --- | --- |
| Asset ID | ____________________ | PM ID | TRI-LAK-MON |
| Equipment | WeighTech MicroWeigh Triple Indexer | Frequency | Monthly |
| Area / Dept | B20 Lakeville · 7340 Maintenance | Date | ____________________ |
| Technician | ____________________ | Hours | ____________________ |

SAFETY — LOTO before any panel work or anything that moves a gate. Menu reads are safe while running. If the scale is sealed for trade, seal check applies to calibration and any sealed parameter.

Source: WeighTech MicroWeigh Triple Indexer manual (46 pp., 2026-04-26) unless the Ref column names another document. "Invertek" = Optidrive ODE-3 User Guide. Frequency assigned by Buddy's Kitchen — WeighTech specifies none.

1  Indicator readings — all three heads

Info menu. Record every value; these only mean something as a trend.

| Reading | Spec | Master | Remote 1 | Remote 2 |
| --- | --- | --- | --- | --- |
| Excite | About 4.5 V. Below 4.0 V blocks calibration; below 1 V = short. |  |  |  |
| S1+ | About half of excite. |  |  |  |
| S1− | Within 0.5 V of S1+. Further apart = miswired or bent cell. |  |  |  |
| ADC | Drift no more than 100–300 counts. No load: within ±10,000 of zero. |  |  |  |
| Offset | Record. |  |  |  |
| Deadload | Record. A climb means build-up in the hopper. |  |  |  |
| Batt | 6 to 8 V. |  |  |  |
| Stbl err | Record, then reset. |  |  |  |
| Dump err | Record, then reset. |  |  |  |
| Audit cfg | Must match last month unless a work order explains it. |  |  |  |
| Audit cal | Must match last month unless a calibration was done. |  |  |  |

2  Load cell test (no recalibration)

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Enter Calibrate on each head, then press Cancel at the cal weight prompt. | Exits without calibrating. Manual offers this as a load cell test. | §11.5 p26 |  |
| 2 | Note any warning raised on entry. | None. Possible: Check load cell / Check excite / Check ex− wire / Check ex+ wire. | §9.2 p17 |  |

3  Communications

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | RS-485 menu — record 485 crc: ________ 485 err: ________ | Flat vs last month. Rising = comm problem. | §10.8 p22 |  |
| 2 | Confirm no comm banner seen since last PM. | No Rmt 1 / Rmt 2 / Mstr comm fail? | §9.1 p17 |  |

4  Gate and valve test

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Select Cycle off. | All valves energize — every gate shuts. | §12.3 p28 |  |
| 2 | Select Cycle idle. | All valves de-energize — every gate opens. | §12.3 p28 |  |
| 3 | Note any gate that does not move on either step. | All move. If one does not, swap parts to isolate. | §12.3 p28 |  |

5  I/O test  (Setup menu → I/O test)

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Toggle Line — infeed belt. | Belt runs / stops to match. | §10.7 p22 |  |
| 2 | Toggle Master dvrt and Master wgt. | Gate follows the command. | §10.7 p22 |  |
| 3 | Toggle Rmt #1 dvrt and Rmt #1 wgt. | Gate follows the command. | §10.7 p22 |  |
| 4 | Toggle Rmt #2 dvrt and Rmt #2 wgt. | Gate follows the command. | §10.7 p22 |  |
| 5 | Toggle Index belt. | Belt runs / stops to match. | §10.7 p22 |  |
| 6 | Watch Opto live display while breaking the beam. | Box / no box changes. | §10.7 p22 |  |
| 7 | For every point above: does the display match what the hardware actually did? | Yes. A mismatch = plumbing reversed, or wrong Opto mode setting. | §6.1 p9 |  |

6  Function tests  (Info menu)

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Force index — index one box without dumping. | Box stops under the hopper funnel. | §11.6 p27 |  |
| 2 | Wiggle test — force the diverters to wiggle. | Diverter gates move. | §10.9 p23 |  |
| 3 | Fill jam — read jam detection status. | Clear. | §10.9 p23 |  |

7  Drives — quick check  (both drives)

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Heatsink fans spin freely and are dust free. Cooling air clear. | No dust mat on the fins. | Invertek 3.9 |  |
| 2 | P0-23 (heatsink >85 °C) and P0-24 (internal >80 °C). | BOTH must read 0:00:00. Anything else = cooling problem, chase it now. | §13/§14 |  |
| 3 | P0-13 trip log — 4 deep. | No new fault since last PM. Copy any entry onto the work order. | §13/§14 |  |
| 4 | Record P0-09 heatsink temp — Infeed: ______ Takeaway: ______ | As-built 22 C / 23 C. | §13/§14 |  |
| 5 | Record P0-20 internal temp — Infeed: ______ Takeaway: ______ | As-built 35 C / 36 C. | §13/§14 |  |

8  Totals

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Record Cnt / Wgt / Avg for all hoppers, then clear if the site wants per-period figures. | Clear totals asks to confirm before clearing. | §10.3 p19 |  |

| Findings / work orders raised: | Technician signature: Date completed: |
| --- | --- |