SEMI-ANNUAL PM   MicroWeigh Triple Indexer

Buddy's Kitchen — Lakeville, MN   ·   Approx. 4–5 hr · machine down, LOTO

| Field | Value | Field | Value |
| --- | --- | --- | --- |
| Asset ID | ____________________ | PM ID | TRI-LAK-SEMI |
| Equipment | WeighTech MicroWeigh Triple Indexer | Frequency | Semi-annual (2× / year) |
| Area / Dept | B20 Lakeville · 7340 Maintenance | Date | ____________________ |
| Technician | ____________________ | Hours | ____________________ |

SAFETY — LOTO before any panel work or anything that moves a gate. Menu reads are safe while running. If the scale is sealed for trade, seal check applies to calibration and any sealed parameter.

Do the Monthly PM first, or on the same day — this sheet does not repeat it.

Source: WeighTech MicroWeigh Triple Indexer manual (46 pp., 2026-04-26) unless the Ref column names another document. "Invertek" = Optidrive ODE-3 User Guide. Frequency assigned by Buddy's Kitchen — WeighTech specifies none.

1  Calibration — all three heads

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Record Audit cal and Audit cfg BEFORE starting. cal ______ cfg ______ | Match last month's record, or a work order explains it. | §3.6–3.7 p6 |  |
| 2 | Menu → Calibrate → password. Default is Zero, Zero, Zero. |  | §7.1 p13 |  |
| 3 | Key in cal weight size in pounds. | 1, 2, 5 or 10 lb. | §7.2 p13 |  |
| 4 | Zero: remove all weight and foreign objects, let vibration die out, press Enter. | Zero accepted. | §7.4 p14 |  |
| 5 | Apply the cal weight and accept. | Accepted with no warning. | §7.5 p14 |  |
| 6 | Repeat for all three heads. Record Audit cal after. after ______ | Incremented by one per calibration. | §3.7 p6 |  |
| 7 | If the scale is sealed for trade: seal check and re-seal per site procedure. | Seal intact and documented. | site proc. |  |

2  Load cells and cabling

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Re-read Excite / S1+ / S1− after calibration, all three heads. | Excite ~4.5 V. S1+ and S1− each about half of excite, within 0.5 V of each other. | §12.1 p28 |  |
| 2 | Load cell cable at each head — no chafe, cuts or crushing. | Jacket intact the full run. | §12.1 p28 |  |
| 3 | Strain relief EF0009 at each head — tight, gland sealed. | No movement at the entry. | §15 p39 |  |
| 4 | Compare Deadload against the 6-month trend. | Flat. Climbing = build-up on the weigh hopper, or a bending cell. | §11.6 p27 |  |

3  Indicator housings

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Main gasket WE0028 on each head — seated, not cracked or flattened. | Sealed all round. A leaking housing is how these fail. | §15 p39 |  |
| 2 | Housing screws HW0018 / HW0019 — all present and tight. | None missing. | §15 p39 |  |
| 3 | Display readable; adjust contrast if washed out. | Crisp at normal viewing angle. | §11.4 p25 |  |
| 4 | Clock — check date and time on each head. | Correct. Clock is battery-backed and keeps time powered off; a clock that lost time points at the battery. | §11.4 p25 |  |

4  Box-detect sensor pair

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Clean emitter (SE0066) and receiver (SE0065) lenses. | Clean and dry. | §15 p39 |  |
| 2 | Check alignment across the full box path on the Opto live display. | Solid, repeatable detection end to end. | §10.7 p22 |  |
| 3 | Cordset SE0038 connector and gland — dry, undamaged. | No moisture ingress. | §15 p39 |  |
| 4 | IF REWIRING WITH POWER ON: connect the brown wire (12 V) LAST. | Brown/blue swapped, or blue connected last, puts the sensor in alarm mode — it LOOKS like it works, LEDs and all, but the controller gets the wrong signal. | §12.2 p28 |  |

5  Drives — full service  (both drives)

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Ambient temperature around the drives. | −10 to 50 °C open (IP20) / −10 to 40 °C enclosed. 95% RH non-condensing. Frost and condensation free. | Invertek 9.1 |  |
| 2 | Heatsink fans free and dust free. | Spin freely by hand, no dust mat. | Invertek 3.9 |  |
| 3 | Enclosure free of dust and condensation; ventilation fans and filters flowing. | Clean and dry inside. | Invertek 3.9 |  |
| 4 | Torque-check drive terminals. | Control 0.5 N·m (4.5 lb-in). Power 1 N·m (9 lb-in). Frame sizes 1–3. | Invertek 3.3/3.5 |  |
| 5 | Inspect power cables for heat damage. | No discoloration, embrittlement or deformation. | Invertek 3.9 |  |
| 6 | Record the full P0 set and compare against the as-built listing — see reverse. | Every value matches, or the difference is written on the work order. | §13 p31 / §14 p35 |  |

| Param | Parameter | As-built Infeed | As-built Takeaway | Infeed today | Takeaway today |
| --- | --- | --- | --- | --- | --- |
| P0-09 | Heatsink temperature | 22 C | 23 C |  |  |
| P0-10 | Run time since manufacture | 0:00:01 | 0:00:01 |  |  |
| P0-13 | Trip log (4 deep) | No Fault ×4 | No Fault ×4 |  |  |
| P0-20 | Internal drive temperature | 35 C | 36 C |  |  |
| P0-23 | Accum. heatsink > 85 C | 0:00:00 | 0:00:00 |  |  |
| P0-24 | Accum. internal > 80 C | 0:00:00 | 0:00:00 |  |  |
| P0-26 | kWh meter | 0.0 kWh | 0.0 kWh |  |  |
| P0-27 | Drive fan run time | 0:00:18 | 0:00:22 |  |  |
| P0-28 | Software version | V3.11 72B9 | V3.11 72B9 |  |  |
| P0-30 | Drive serial number | 649453/18/057 | 654310/24/083 |  |  |
| P0-33 | Fault counter O-I | 0 | 0 |  |  |
| P0-34 | Fault counter O-Volts | 0 | 0 |  |  |
| P0-35 | Fault counter U-Volts | 1 | 1 |  |  |
| P0-36 | Fault counter O-temp | 0 | 0 |  |  |
| P0-39 | Modbus comms errors | 4 | 4 |  |  |
| P0-40 | CANbus comms errors | 255 | 255 |  |  |

6  Air system and pneumatics

The manual: "Most machine related problems can be traced to bad air supply (excessive water and condensation), sticky air cylinders, worn or clogged solenoid air valves, and wiring." — §12.3 p28

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Drain condensate; check / replace the filter element. | No free water carried downstream. | §12.3 p28 |  |
| 2 | Regulator AA0009 at setpoint. | Setpoint: ____________ (confirm with WeighTech — not in the manual). | §15 p39 |  |
| 3 | Tubing AA0006 — no stiffness, cracks or kinks. Fitting seats clean. | Flexible, seated. | §15 p39 |  |
| 4 | Cylinder rods AC0001 (weigh hoppers) and AC0002 (diverters) — clean, unscored. | No scoring. No slow, sticky or hesitant motion on any gate. | §12.3 p28 |  |
| 5 | Rod clevis AA0026 and pins — check for play. | No slop at the pin. | §15 p39 |  |
| 6 | Valve manifold — DIN plug LEDs (AV0045) lit; no leaks at substation or end plate. | All LEDs lit, no audible leak, no buzzing valve. | §12.3 p28 |  |
| 7 | Flow controls AA0005 — settings not drifted from where they were set. | Gate speed unchanged. | §15 p39 |  |
| 8 | Air quality — confirm residual oil against the compressor oil in use. | Mineral oil / PAO: max 5 mg/m³ (ISO 8573-1 Cl. 4). Bio-oil: max 0.1 mg/m³ (Cl. 2). Above this the valve's lubricant flushes out. | Festo |  |

7  Drive train

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Sprockets 11377 and CB0117 — tooth wear. | No hooked or thinned teeth. | §15 p39 |  |
| 2 | Belting CB0060 (buffer) and CB0126 (takeaway flights) — tracking and condition. | Tracking centered, no torn or missing flights, no edge damage. | §15 p39 |  |
| 3 | Motor 10575 mounts and bearings. | Mounts tight, no bearing noise or shaft play. | §15 p39 |  |

8  Stop circuit

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Function-test stop switch SS0006 and pushbuttons SS0020 (green) / SS0024 (red). | Machine stops on demand from every station. | §15 p39 |  |

9  Records

| # | Task | Spec / what good looks like | Ref | ✓ |
| --- | --- | --- | --- | --- |
| 1 | Record App / Build / Date from each head. | As-built: tri index 3 · Build 49 · 02/13/2024, all three. A mismatch between heads is a finding. | §10.9 p23 |  |
| 2 | Record each indicator serial from the front panel. | Not printed in the manual — front panel only. WeighTech asks for it on every support call. | §12.5 p30 |  |
| 3 | Back up Parameters and Dump settings; verify drive parameters against the §13/§14 listings. Attach to the asset record. | Current archive on file, dated today. | §10.5–10.6 |  |
| 4 | Reconcile spares on hand against the parts list. | At or above min for every stocked line. | §15 p39 |  |

| Findings / work orders raised: | Technician signature: Date completed: |
| --- | --- |